Ericsson SIMS — Technical Interview Practice

# L2 MAC Frame Parser

Topic: C++ · L2/MAC protocol parsing · Bit manipulation · Error handling

## Phase 1 — Code Review  (~20 min)

Read the following buggy implementation. Find all bugs and quality issues before looking at the answers.

Code to review

// mac_parser.cpp #include <cstdint> enum class FrameType { MAC = 0, RLC = 1, PDCP = 2 }; struct L2Header { uint8_t version; uint8_t type; uint16_t length; uint32_t seq_num; }; // Parses 7 bytes: [ver¦type][len_hi][len_lo][seq3][seq2][seq1][seq0] FrameType parse_header(const uint8_t* data, L2Header& header) { header.version = data[0] >> 4; header.type = data[0] & 0xF; header.length = (data[1] << 8) ¦ data[2]; header.seq_num = (data[3] << 24) ¦ (data[4] << 16) ¦ (data[5] << 8) ¦ data[6]; return static_cast<FrameType>(header.type); }

Try to find the bugs yourself first. There are 4 bugs and 3 quality issues.

Answers — Bugs Found

| # | Type | Description |
| --- | --- | --- |
| 1 | BUG | data is never null-checked — dereferencing NULL is undefined behaviour. |
| 2 | BUG | data[1] << 8: uint8_t is promoted to int before shift. On 16-bit platforms this overflows. Cast to uint16_t first. |
| 3 | BUG | data[3] << 24: shifting a signed int left by 24 bits can overflow into the sign bit — undefined behaviour. Must cast to uint32_t. |
| 4 | BUG | static_cast<FrameType>(header.type): if type is 3–15 there is no matching enum value — undefined behaviour. Validate before casting. |
| 5 | QUALITY | No check that buffer is at least 7 bytes — caller could pass a 3-byte buffer causing out-of-bounds read. |
| 6 | QUALITY | No error return path — function returns FrameType but has no way to signal parse failure. |
| 7 | QUALITY | Endianness assumption is undocumented — big-endian byte order assumed but never stated. |

## Phase 2 — Test Cases  (~20 min)

Write test cases that catch every bug found in Phase 1. Cover: valid frames, invalid type, null pointer, short buffer, max values.

#include <cassert> #include <cstdio> #include "mac_parser.hpp" // Test 1 — valid MAC frame, all fields parsed correctly void test_valid_mac_frame() { // ver=1, type=0(MAC), len=100, seq=1 uint8_t buf[] = {0x10, 0x00, 0x64, 0x00, 0x00, 0x00, 0x01}; L2Header hdr{}; auto result = parse_header(buf, sizeof(buf), hdr); assert(result.has_value()); assert(*result == FrameType::MAC); assert(hdr.version == 1); assert(hdr.length == 100); assert(hdr.seq_num == 1); printf("PASS test_valid_mac_frame\n"); } // Test 2 — valid RLC frame void test_valid_rlc_frame() { uint8_t buf[] = {0x21, 0x01, 0x00, 0x00, 0x00, 0x00, 0x05}; L2Header hdr{}; auto result = parse_header(buf, sizeof(buf), hdr); assert(result.has_value()); assert(*result == FrameType::RLC); printf("PASS test_valid_rlc_frame\n"); } // Test 3 — invalid type (15) must return error void test_invalid_frame_type() { uint8_t buf[] = {0x0F, 0x00, 0x10, 0x00, 0x00, 0x00, 0x00}; L2Header hdr{}; auto result = parse_header(buf, sizeof(buf), hdr); assert(!result.has_value()); printf("PASS test_invalid_frame_type\n"); } // Test 4 — null pointer must not crash void test_null_pointer() { L2Header hdr{}; auto result = parse_header(nullptr, 7, hdr); assert(!result.has_value()); printf("PASS test_null_pointer\n"); } // Test 5 — buffer too short must return error void test_buffer_too_short() { uint8_t buf[] = {0x10, 0x00, 0x64}; L2Header hdr{}; auto result = parse_header(buf, sizeof(buf), hdr); assert(!result.has_value()); printf("PASS test_buffer_too_short\n"); } // Test 6 — max values: length=0xFFFF, seq=0xFFFFFFFF void test_max_values() { uint8_t buf[] = {0x10, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF}; L2Header hdr{}; auto result = parse_header(buf, sizeof(buf), hdr); assert(result.has_value()); assert(hdr.length == 0xFFFF); assert(hdr.seq_num == 0xFFFFFFFF); printf("PASS test_max_values\n"); } int main() { test_valid_mac_frame(); test_valid_rlc_frame(); test_invalid_frame_type(); test_null_pointer(); test_buffer_too_short(); test_max_values(); printf("All tests passed.\n"); }

## Phase 3 — Implementation  (~25 min)

Write the correct implementation from scratch. Fix all bugs. Use std::optional for error signalling.

// mac_parser.hpp — correct version #pragma once #include <cstdint> #include <optional> enum class FrameType : uint8_t { MAC = 0, RLC = 1, PDCP = 2 }; struct L2Header { uint8_t version; uint8_t type; uint16_t length; uint32_t seq_num; }; static constexpr std::size_t L2_HEADER_SIZE = 7u; std::optional<FrameType> parse_header( const uint8_t* data, std::size_t len, L2Header& header) { if (data == nullptr ¦¦ len < L2_HEADER_SIZE) return std::nullopt; header.version = static_cast<uint8_t>(data[0] >> 4u); header.type = static_cast<uint8_t>(data[0] & 0x0Fu); // Validate type before casting to enum if (header.type > static_cast<uint8_t>(FrameType::PDCP)) return std::nullopt; // Cast to wider type BEFORE shifting to avoid UB header.length = static_cast<uint16_t>( (static_cast<uint16_t>(data[1]) << 8u) ¦ static_cast<uint16_t>(data[2])); header.seq_num = (static_cast<uint32_t>(data[3]) << 24u) ¦ (static_cast<uint32_t>(data[4]) << 16u) ¦ (static_cast<uint32_t>(data[5]) << 8u) ¦ static_cast<uint32_t>(data[6]); return static_cast<FrameType>(header.type); }

Follow-up questions — be ready to answer

- Why std::optional instead of returning -1 or throwing? — No exception overhead, caller is forced to check, explicit API contract.

- What is integer promotion? — uint8_t promotes to int before arithmetic. Dangerous on shifts if int is narrow.

- If this ran on a little-endian CPU reading a big-endian wire format, what breaks? — length and seq_num would be byte-swapped.

- How would you make this thread-safe? — Function is already stateless/reentrant — safe as-is if each thread has its own hdr.